eta

Tutorial TWIST MMC

Table of contents

Introduction	4

Additional environment setup	4

MMC structure	4

Using Scope Mimicry library	5

Blinky MMC arm	5

Objectives	5

Required Hardware	5

Required Software	5

Pull the project in VScode from GitHub	5

Code explanation	7

Hardware setup	13

Executing the tutorial	14

Sorting algorithm (CVB)	16

Objectives	16

Required Hardware	17

Required Software	17

Pull the project in VScode from GitHub	17

Code explanation	19

Hardware setup	22

Executing the tutorial	23

Individual MMC module test	24

Objectives	24

Required Hardware	26

Required Software	26

Pull the project in VScode from GitHub	26

Code explanation	28

Hardware setup	30

Executing the tutorial	33

# Introduction

The goal of this tutorial is to use the OnwTech TWIST boards to build an MMC progressively. Please refer to the “Tutorial_Twist_Board” for learning how to use the Twist board before starting this new series of tutorials.

## Additional environment setup

After the basic environment setup on “Tutorial_Twist_Board”, you will also need to:

- Install Git graph in VScode.

- Follow this tutorial to create a SSH key and add it to your GitHub account: https://medium.com/@techworldthink/a-beginners-guide-to-adding-ssh-key-to-github-windows-6a5de5396ba6

- If your PC user is not administrator, you will need to modify the file in which you will save the SSH key in Step1:

Enter file in which to save the key (C:\Users\admin/.ssh/id_rsa): C:\Users\your-user-name/.ssh/id_rsa

Then, you will probably have this warning when trying to follow Step 3 in the tutorial:

@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@

@         WARNING: UNPROTECTED PRIVATE KEY FILE!          @

@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@

Permissions for 'C:\\Users\\your-user-name/.ssh/id_rsa' are too open.

It is required that your private key files are NOT accessible by others.

This should concern you if you share your PC with other people, if it is the case, you should get in touch with the IT team of the lab.

## MMC structure

Write here about difference between the classical MMC structure and what we want to do in the CARROTS project.

## Using Scope Mimicry library

# Blinky MMC arm

## Objectives

The goal of this tutorial is to do a first MMC arm without using power with the following structure:

The main board serves as the central controller in the classical structure of the MMC. It will generate a connection sequence going from 0 to 3 that indicates how many modules should be connected. Then, it choses which modules to connect according to the preset preference order M1 → M2 → M3.

Then, the main board sends the connection commands g1, g2 and g3 via the Ethernet cables to the M1, M2 and M3 boards. If the board receives the command to be connected, it turns ON its SPIN board LED. If the board receives the command to be disconnected, it turns OFF its SPIN board LED.

## Required Hardware

- 4 TWIST boards with SPIN

- DC Power Supply (48 V, 2 A)

- 3 Ethernet cables (RJ45)

- Cables to connect with the power supply

- PC 64-bits (windows or linux)

## Required Software

- Git

- Visual Studio Code with PlatformIO

## Pull the project in VScode from GitHub

First, we need to load the Blinky MMC arm code from the “MMC_merged” branch in https://github.com/Ayoub-Farah/Hackathon_toulouse/ .

- Open Git graph window in VScode by clicking in the button marked in red:

- In the Git graph window, select the  icon and to set the Remote Configurations compatible to the remote repository you want to pull from.

- Click on Add Remote and add a new remote repository called hackathon with Fetch URL equal to https://github.com/Ayoub-Farah/Hackathon_toulouse

- In Git Graph window, click right on the branch and checkout to this branch keeping the same name for the local branch. In this tutorial it is the MMC_merged branch.

- Now you have the good code to execute the Blinky MMC arm in your VScode.

## Code explanation

- Starting by the setup_routine function, it configures the peripherals and main loop_functions:

It first configures the SPIN LED by using a function coded in Low Level.

config_led_LL(); // Configure the LED pin in Low Level

Then, it sets the TWIST board to Buck mode, declares the background, communication and critical tasks and starts them. The critical task has a frequency of 10 kHz.

After that, it configures the communication protocol used by informing the transmitted and reception buffer variables, the function that gives the actions of the board when it receives a message and the transmission speed.

communication.rs485.configure(buffer_tx, buffer_rx, sizeof(buffer_rx),

reception_function,

SPEED_20M); // custom configuration for RS485

Finally, it configures a some scope channels for the master to acquire the connection sequence generated “N_u” and the connection signal for each module “g1”, “g2” and “g3”.

- Continuing to the loop_communication_task function, it makes the main board change between the IDLE and POWER modes, and enables scope data triggering and recording according to the pressed key in the keyboard.

IDLE mode is equivalent to key “i” and POWER mode is equivalent to key “p”.

Data trigger becomes equal to true when the key ‘a’ is pressed, setting the starting time of data acquisition. It should be pressed during POWER mode.

bool a_trigger()

{

return enable_acq;

}

The data recording function dump_scope_datas stores the acquired data in both csv and text format in the folder “Data_records” within “src” when the key “r” is pressed. It happens only in IDLE mode.

void dump_scope_datas(ScopeMimicry &scope)

{

uint8_t *buffer = scope.get_buffer();

/* We divide by 4 (4 bytes per float data) */

uint16_t buffer_size = scope.get_buffer_size() >> 2;

printk("begin record\n");

printk("#");

for (uint16_t k = 0; k < scope.get_nb_channel(); k++)

{

printk("%s,", scope.get_channel_name(k));

}

printk("\n");

printk("# %d\n", scope.get_final_idx());

for (uint16_t k = 0; k < buffer_size; k++)

{

printk("%08x\n", *((uint32_t *)buffer + k));

task.suspendBackgroundUs(100);

}

printk("end record\n");

}

- Next, the loop_background_task function makes the LED toggle or turn off according if it is in IDLE or POWER mode and it prints some variables values from the control part.

- Finally, the loop_critical_task function contains the whole communication and control logic.

If the board is the Main (module_ID == MMC_LEAD):

In POWER mode, the control part generates the connection sequence using the order of variable seq_u = {1, 2, 3, 2, 1, 0}. The sequence has a certain period sw_period that regulates the width of the staircase step.

if (sw_timer == sw_period)

{

if (counter_seq >= 6)

{

counter_seq = 0;

}

number_of_connected_submodules_upper_arm = (float)seq_u[counter_seq]; // recuperate as float for scope

counter_seq++;

sw_timer = 0;

}

sw_timer++;

After that, the connection variables are modified to 0 or 1 according to the connection priority M1→M2→M3.

if (number_of_connected_submodules_upper_arm == 0)

{

g[0] = 0; // g[0] -> M1

g[1] = 0; // g[1] -> M2

g[2] = 0; // g[2] -> M3

}

if (number_of_connected_submodules_upper_arm == 1)

{

g[0] = 1;

g[1] = 0;

g[2] = 0;

}

if (number_of_connected_submodules_upper_arm == 2)

{

g[0] = 1;

g[1] = 1;

g[2] = 0;

}

if (number_of_connected_submodules_upper_arm == 3)

{

g[0] = 1;

g[1] = 1;

g[2] = 1;

}

g_u_1 = (float)g[0]; // recuperate as float for scope

g_u_2 = (float)g[1]; // recuperate as float for scope

g_u_3 = (float)g[2]; // recuperate as float for scope

And the scope acquires data according to a certain period equal to tacq=scopeperiod⋅criticalperiod.

if (scope_timer == scope_period)

{

scope.acquire();

scope_timer = 0;

}

scope_timer++;

In terms of communication, the main board saves in the structure dataTX_mmc:

- The commands corresponding to each module.

- Its own ID to identify from who the message is coming.

- Status saying if it is in POWER mode = 1 or in IDLE mode = 0.

And then starts the communication to send these information to the modules boards.

SET_SIGNAL(dataTX_mmc.command, MMC_SM1, g[0]);

SET_SIGNAL(dataTX_mmc.command, MMC_SM2, g[1]);

SET_SIGNAL(dataTX_mmc.command, MMC_SM3, g[2]);

dataTX_mmc.ID = module_ID;

memcpy(buffer_tx, &dataTX_mmc, sizeof(dataTX_mmc));

dataTX_mmc.status = 1;

communication.rs485.startTransmission();

We can see that the dataTX_mmc and dataRX_mmc structures have in its definition one more information to store the capacitor voltage of the modules.

struct MMC_frame

{

uint8_t command;

float32_t Capacitor_Voltage;

uint8_t status;

uint8_t ID;

} __packed;

If it enter IDLE mode, the Main sends an IDLE flag to all modules to enter IDLE mode as well via the status information.

else if (mode == IDLEMODE)

{

/* Made to send IDLE flag only once */

if (!send_idle)

{

dataTX_mmc.ID = module_ID;

dataTX_mmc.status = 0;

memcpy(buffer_tx, &dataTX_mmc, sizeof(dataTX_mmc));

communication.rs485.startTransmission();

send_idle = true; // Set the flag to send idle command

}

}

In the reception_function we can see that the main board receives an artificial information of capacitor_voltage from the modules and stores it in a the vector MMC_capacitor_voltage. The capacitor_voltage information will be later substituted by the measure of the capacitor voltage acquired by the board.

If the board is a module (module_ID == MMC_SMx):

Analysing reception_function, it receives the information from the other boards and:

- Verify if the sender is the main if (dataRX_mmc.ID == MMC_LEAD):

  - Stores the command signal in module_comand.

  - Changes to POWER mode or IDLE mode according to the main board status.

- Verify if it is the next one in line to send the next message if ((dataRX_mmc.ID == module_ID - 1)):

  - Updates the transmission data structure for its own values of ID and capacitor voltage.

  - And then starts the communication to send these information to the other boards only if in POWER mode.

if (dataRX_mmc.ID == MMC_LEAD)

{

/* retrievig command from lead message*/

module_comand = GET_SIGNAL(dataRX_mmc.command, module_ID);

/* retrieving status */

if (dataRX_mmc.status == 1)

{

mode = POWERMODE;

}

else

{

mode = IDLEMODE;

}

}

/* The board following the ID of the one who sent will start sending

the next message */

if ((dataRX_mmc.ID == module_ID - 1))

{

dataTX_mmc = dataRX_mmc; // Copy the received data to the transmission data

dataTX_mmc.ID = module_ID;

dataTX_mmc.Capacitor_Voltage = MMC_voltage; /* TODO :uncomment when we get the voltage */

if (mode == POWERMODE)

{

memcpy(buffer_tx, &dataTX_mmc, sizeof(dataTX_mmc));

communication.rs485.startTransmission();

}

Observe that the sending order goes as follows:

- Main

- SM3

- SM2

- SM1

This happens since the IDs are defined as:

#define MMC_LEAD 0

#define MMC_SM1 1

#define MMC_SM2 2

#define MMC_SM3 3

}

In the critical function, while in POWER mode a module board:

- Turns ON its SPIN LED if it is commanded to be connected by the Main (module_command = 1)

- Turns OFF its SPIN LED if it is commanded to be disconnected (module_command =0).

else

{

if (module_comand != module_command_past)

{

change_state_command = true; // Set the flag to change the state

}

if (module_comand)

{

if (change_state_command)

{

Led_turnON_LL();

change_state_command = false; // Reset the flag

}

}

else

{

if (change_state_command)

{

Led_turnOFF_LL();

change_state_command = false; // Reset the flag

}

}

}

module_command_past = module_comand; // Update the past command

## Hardware setup

In terms of hardware, you will connect the 4 boards as in the image:

You have to upload in each board the main.ccp code changing the variable module_ID to MMC_LEAD, MMC_SM1, MMC_SM2 or MMC_SM3 according to the module function.

/* --------------USER VARIABLES DECLARATIONS------------------- */

/* TODO : Define module_ID depending on the ID of the board */

uint8_t module_ID = MMC_SM1; // The ID of the module, can be set to MMC_LEAD or any other SMx

After making the connections and connecting the PC to the main with the USB-C cable, you should have something like this:

We connect the boards in series to and set the power supply to 15 V in order to power their SPIN boards.

## Executing the tutorial

After uploading the code to all three boards, open the serial monitor using the  icon while connected with the master, as shown in the hardware setup.

- Press h to display the help menu.

- The main and M1, M2 and M3 boards starts in IDLE mode.

- Turn on the power supply around 15 V to power the 4 SPIN boards.

Return to step 3 to execute it again

Press p to switch to POWER mode and the modules LEDs will start to light up and off according to the connection sequence.

- Turn off the power supply.

- Press i to switch to IDLE mode and stop the arm operation.

While in POWER mode, you should expect a result like this:

# Sorting algorithm (CVB)

## Objectives

The goal of this tutorial is test the logic of the Capacitor Voltage Balancing (CVB) algorithm. The CVB algorithm is used in the MMC in order to create a preference order for the connection of modules into the arm.

This algorithm is extremely important since the module main element, the capacitor charges and discharges according to the current as seen in the figure:

Therefore, if the connection order is random, the voltage of the modules in one arm are going to have different values every time, which is not good for the converter operation. So, we use the CVB algorithm with the objective of maintaining the capacitors voltages of all modules within one arm in the same mean level.

The algorithm works as follows:

- The algorithm receives the number of modules to be connected N from the modulation block.

- It verifies if this number is different from 0.

- If the arm current iarm is positive, the capacitors are going to be charged. Then, the modules with smaller voltage are connected such that their voltage will increase.

- If the arm current iarm is negative, the capacitors are going to be discharged. Then, the modules with higher voltage are connected such that their voltage will decrease.

To do this, we need to first sort the modules according to their capacitor voltages and then pass through the algorithm that will generate the switching signals indicating if they are connected or not.

## Required Hardware

- 1 TWIST boards with SPIN

- PC 64-bits (windows or linux)

## Required Software

- Git

- Visual Studio Code with PlatformIO

## Pull the project in VScode from GitHub

First, we need to load the Sorting algorithm code from the “FirmwareCVBgate” branch in https://github.com/Ayoub-Farah/Hackathon_toulouse/ .

- Open Git graph window in VScode by clicking in the button marked in red:

- In the Git graph window, select the  icon and to set the Remote Configurations compatible to the remote repository you want to pull from.

If you have already done it for Blinky MMC arm tutorial, skip to step 4.

- Click on Add Remote and add a new remote repository called hackathon with Fetch URL and Push URL equal to https://github.com/Ayoub-Farah/Hackathon_toulouse

- In Git Graph window, click right on the branch and checkout to this branch keeping the same name for the local branch. In this tutorial it is the FirmwareCVBgate branch.

- Now you have the good code to execute the CVB code in your VScode.

## Code explanation

- Starting by the setup_routine function, it configures the peripherals and main loop_functions:

It sets the TWIST board legs to Buck mode, declares the background, communication and critical tasks and starts them. The critical task has a frequency of 10 kHz.

Finally, it configures a some scope channels for the master to acquire the connection sequence generated “N_u” and the connection signal for each module “g1”, “g2” and “g3”.

- Continuing to the loop_communication_task function, it makes the main board change between the IDLE and POWER modes, and enables scope data triggering and recording according to the pressed key in the keyboard. If you have already seen it in Blinky MMC arm tutorial, skip directly to step 4 that describes the critical_task function.

IDLE mode is equivalent to key “i” and POWER mode is equivalent to key “p”.

Data trigger becomes equal to true when the key ‘a’ is pressed, setting the starting time of data acquisition. It should be pressed during POWER mode.

bool a_trigger()

{

return enable_acq;

}

The data recording function dump_scope_datas stores the acquired data in both csv and text format in the folder “Data_records” within “src” when the key “r” is pressed. It happens only in IDLE mode.

void dump_scope_datas(ScopeMimicry &scope)

{

uint8_t *buffer = scope.get_buffer();

/* We divide by 4 (4 bytes per float data) */

uint16_t buffer_size = scope.get_buffer_size() >> 2;

printk("begin record\n");

printk("#");

for (uint16_t k = 0; k < scope.get_nb_channel(); k++)

{

printk("%s,", scope.get_channel_name(k));

}

printk("\n");

printk("# %d\n", scope.get_final_idx());

for (uint16_t k = 0; k < buffer_size; k++)

{

printk("%08x\n", *((uint32_t *)buffer + k));

task.suspendBackgroundUs(100);

}

printk("end record\n");

}

- Next, the loop_application_task function makes the LED toggle or turn off according if it is in IDLE or POWER mode and it prints some variables values from the control part in the serial monitor terminal.

- Finally, the loop_critical_task function contains the whole CVB algorithm logic.

In IDLE mode, we ensure that its PWM deactivated.

In POWER mode, the connection sequence logic is generated as in the Blinky MMC arm. But we add an additional sequence for the lower arm that is complementary from the upper arm sequence.

if (sw_timer == sw_period)

{

if (counter_seq >= 6) {

counter_seq = 0;

}

number_of_connected_submodules_upper_arm = (float)seq_u[counter_seq];  // recuperate as float for scope

number_of_connected_submodules_lower_arm = (float)seq_l[counter_seq];  // recuperate as float for scope

counter_seq++;

sw_timer = 0;

}

Then the sorting algorithm function is executed:

It firsts sorts the modules indexes according to capacitor voltage for both arms. This is done in a while loop within 10 times because this was the necessary amount of loops to sort up to 6 modules.

void sorting()

{

uint8_t counter_loops_sorting = 0;

while(counter_loops_sorting < 10){ // Sorts modules indexes according to capacitor voltage

for(uint8_t counter = 0; counter < total_number_of_modules_arm-1; counter++)

{

if(modules_capacitor_voltages_upper_arm[counter] > modules_capacitor_voltages_upper_arm[counter + 1])

{

float32_t temp = modules_capacitor_voltages_upper_arm[counter];

modules_capacitor_voltages_upper_arm[counter] = modules_capacitor_voltages_upper_arm[counter + 1];

modules_capacitor_voltages_upper_arm[counter + 1] = temp;

float32_t temp2 = modules_indexes_upper_arm[counter];

modules_indexes_upper_arm[counter] = modules_indexes_upper_arm[counter + 1];

modules_indexes_upper_arm[counter + 1] = temp2;

}

if(modules_capacitor_voltages_lower_arm[counter] > modules_capacitor_voltages_lower_arm[counter + 1])

{

float32_t temp = modules_capacitor_voltages_lower_arm[counter];

modules_capacitor_voltages_lower_arm[counter] = modules_capacitor_voltages_lower_arm[counter + 1];

modules_capacitor_voltages_lower_arm[counter + 1] = temp;

float32_t temp2 = modules_indexes_lower_arm[counter];

modules_indexes_lower_arm[counter] = modules_indexes_lower_arm[counter + 1];

modules_indexes_lower_arm[counter + 1] = temp2;

}

}

counter_loops_sorting++;

}

Then, a for loop choses the modules to connect according to the input current and generates the connection switching signal for all modules within the upper and lower arms.

if(counter < number_of_connected_submodules_lower_arm)

{

if(i_lower_arm>=0)

{

uint8_t index_smallest_voltage_capacitor_lower_arm = modules_indexes_lower_arm[counter];

g_l[index_smallest_voltage_capacitor_lower_arm] = 1;

}

else{

uint8_t higher_index = total_number_of_modules_arm-1-counter;

uint8_t index_highest_voltage_capacitor_lower_arm = modules_indexes_lower_arm[higher_index];

g_l[index_highest_voltage_capacitor_lower_arm] = 1;

}

}

Returning to the critical function, it retrives the switching signals for the scope and the scope acquires data according to a certain period equal to tacq=scopeperiod⋅criticalperiod.

if (scope_timer == scope_period)

{

scope.acquire();

scope_timer = 0;

}

scope_timer++;

## Hardware setup

In terms of hardware, you will connect the board with the PC using a USB to USB-C cable.

## Executing the tutorial

Build   and Upload  the code into the board and open the serial monitor using the  icon.

- Press h to display the help menu.

- The board starts in IDLE mode.

Return to step 3 to execute it again

Press p to switch to POWER mode and the CVB starts to choose the connected modules according to the connection sequence.

- Press a to set the start of acquired data.

- Press r when you think you have sufficient data to record.

- Press i to switch to IDLE mode and stop the CVB operation.

- Data will be recorded in src>>Data_records folder.

By plotting the recorded data, expect seeing something like this:

To have this kind of plot, you can use the python code plot_CVB in src>>Data_records folder.

These results were obtained using modules_capacitor_voltages_upper_arm[3] = {3.0,5.0,4.0} and i_upper_arm = 1 (i≥0) for the upper arm to test the algorithm, so the preference of connection should be gu1→gu3→gu2.

You can plot the same for the lower arm using modules_capacitor_voltages_lower_arm[3] = {3.0,5.0,4.0} and i_lower_arm = -1 (i<0), so the preference of connection should be gu2→gu3→gu1.

You can play with the scope and connection sequence steps frequency or try to modify the number of modules, current signal and capacitor voltages to fully understand the code.

# Individual MMC module test

## Objectives

The goal of this tutorial is to test a individual MMC module operation following the structure:

The board serves as a HB module of the MMC by programming its second leg (LEG2) to be deactivated, having the following circuit. Observe that the HIGH terminals are open (Vhigh to GND).

The tutorial will consist of putting the module in OFF and latter ON state according to the correspondence table:

- ON state: the module is considered to be connected to the circuit and the current pass through its capacitor.

- OFF state: the module is considered to be bypassed from the circuit and the current does not pass through its capacitor.

- BLOCK state: will not be used during the tutorial.

Observe that the module state will basically be controlled using the board first leg (LEG1) switches Q1 and Q2.

A diode will be used for protecting the power supply from currents in the opposite sense. Because of that, the current passing through the module will be always positive and its behavior will be only tested in the condition when the module capacitor is charged.

## Required Hardware

- 1 TWIST boards with SPIN

- DC Power Supply (48 V, 2 A)

- Cables to connect with the power supply

- PC 64-bits (windows or linux)

## Required Software

- Git

- Visual Studio Code with PlatformIO

## Pull the project in VScode from GitHub

First, we need to load the MMC modules test code from the “code_test_mmc” branch in https://github.com/luizvilla/Core .

- Open Git graph window in VScode by clicking in the button marked in red:

- In the Git graph window, select the  icon and to set the Remote Configurations compatible to the remote repository you want to pull from.

- Click on Add Remote and add a new remote repository called test_module_MMC with Fetch URL equal to https://github.com/luizvilla/Core

- In Git Graph window, click right on the branch and checkout to this branch keeping the same name for the local branch. In this tutorial it is the code_test_MMC branch.

- Now you have the good code to execute the module test in your VScode.

## Code explanation

- Starting by the setup_routine function, it configures the peripherals and main loop_functions:

It sets the TWIST board legs to Buck and Boost mode and changes the duty cycle maximum and minimum limits to 1 and 0, respectively. This is necessary to fully control the module switches.

It declares the background, communication and critical tasks and starts them. The critical task has a frequency of 10 kHz.

Finally, it configures a some scope channels.

- Continuing to the loop_communication_task function, it makes the board do the following functions according to the pressed key in the keyboard:

  - Change between the IDLE (i) and POWER (p) modes

  - Increases (u) or decreases (d) the duty cycle if in POWER mode

  - Change to SWITCH mode (s), that is the mode we are using during the tutorial

  - Changes the module state to ON (o), OFF (f), BLOCK (b), only works in SWITCH mode

  - Enables scope recording (r)

The data recording function dump_scope_datas stores the acquired data in both csv and text format in the folder “Data_records” within “src” when the key “r” is pressed. It happens only in IDLE mode.

void dump_scope_datas(ScopeMimicry &scope)

{

uint8_t *buffer = scope.get_buffer();

/* We divide by 4 (4 bytes per float data) */

uint16_t buffer_size = scope.get_buffer_size() >> 2;

printk("begin record\n");

printk("#");

for (uint16_t k = 0; k < scope.get_nb_channel(); k++)

{

printk("%s,", scope.get_channel_name(k));

}

printk("\n");

printk("# %d\n", scope.get_final_idx());

for (uint16_t k = 0; k < buffer_size; k++)

{

printk("%08x\n", *((uint32_t *)buffer + k));

task.suspendBackgroundUs(100);

}

printk("end record\n");

}

- Next, the loop_application_task function makes the LED toggle, turn on or turn off according if it is in IDLE, POWER or SWITCH mode and it prints the measured currents and voltages Ilow1, Vlow1 and Vhigh in the serial monitor terminal.

- Finally, the loop_critical_task function contains the module states logic.

First, the measurements from the boards sensors are used to update the measurement variables.

In the different modes:

- If the board is in IDLE mode, we ensure that its PWM deactivated.

- If the board is in POWER mode, the board LEG1 will work as a Buck with a duty cycle initially equal to 0,3. We will not use this mode.

- If the board is in SWITCH mode, the board will work as a MMC module having the following states:

  - g=0 (OFF): as LEG1 is in buck convention, the duty cycle is set to 0 to open Q1 and close Q2 and enables the PWM.

shield.power.setDutyCycle(LEG1,0.0);

  - g=1 (ON): as LEG1 is in buck convention, the duty cycle is set to 1 to close Q1 and open Q2 and enables the PWM.

shield.power.setDutyCycle(LEG1,1.0);

  - g=2 (ON): opens both MOSFETS Q1 and Q2.

if (pwm_enable == true)

{

shield.power.stop(ALL);

}

pwm_enable = false;

According to documentation, the buck convention uses the PWM channel to control the HIGH switch (Q1) and the boost convention uses the PWM channel to control the LOW switch (Q2).

Finally, the scope acquires the data at each critical loop cycle.

## Hardware setup

- In terms of hardware, you will connect the board with the power supply and oscilloscope as in the image:

More specifically for the Oscilloscope connection, you will need to use a BNC adaptor to the differential voltage you want to measure (VLOW1 to GND)

- Connect the board to the PC via a USB-C cable

- Build and upload the main.ccp code into the board SPIN.

- Configure Ownplot following these steps:

  - Open the Serial Monitor and check the Port Name

  - Close the serial monitor and Launch OwnPlot

  - In the Port tab, select the port connected to your PC (e.g., if the port is COM9, select COM9.)

  - Click the Open Port button  to open the port. It should change to "Open."

  - Set the Number of Channels in the Settings tab to 3.

  - Customize Legends in the Graph tab to Ilow1, Vlow1, Vhigh, in this order.

  - Configure the Commands in the Send tab as in the application task:

    - Idle mode = i

    - Power mode = p

    - Switch mode = s

    - Module ON = o

    - Module off = f

    - Module block = b

    - Download scope data = r

    - Duty_cycle Up = u

    - Duty_cycle Down = d

- Configure your Oscilloscope:

  - Make sure it acquires the output voltage of the module (V1low to GND)

  - Use a 20 ms/div or 10 ms/div configuration.

  - Configure it to trigger when the voltage rises up to 7 V to see how the module voltage changes during the transition from state OFF to ON in the oscilloscope.

  - You can also add a current probe if you would like to measure the circuit current.

In my setup, I used PicoScope 4444 as oscilloscope for voltage measurement.

After making the connections and connecting the PC to board the USB-C cable, you should have something like this:

PC

Resistor

Diode

Oscilloscope

Power supply

## Executing the tutorial

After uploading the code to the board and setting up OwnPlot.

Return to step 4 to execute it again

The board starts in IDLE mode (board disactivated).

- Turn on the power supply around 10 V to power the circuit.

- Press the button “SWITCH mode” and the module will start already in ON module.

- Press the button “Module OFF” to put the module in OFF state.

- Press the button “Module ON” to put the module in ON state to see the transition.

- Press i to switch to IDLE mode and stop the arm operation.

If you acquire the transition in the oscilloscope using the trigger, you should expect a result like this:

In my case, Scope 1 measures the current entering the resistor and Scope 2 measures the voltage between the board terminals Vlow1 and GND, that are the modules terminals. This means that Scope 2 measures the output voltage of the half-bridge module.

The module starts at OFF and, at this moment, the module is bypassed having its voltage VSM=0 V. Therefore, all the power supply voltage is in the resistor and a current of I≈0,4 A circulates into the circuit.

Then, the module transitions from OFF to ON state. Its capacitor Chigh starts to charge until it achieves a voltage of VSM=9,3 V. We would expect it to have is voltage around 10 V during this phase, however, there are additional voltage drops on the resistor and on the board inductor. The current also goes down near to zero.

We also observe a unexpected behavior of the voltage at ON state. It goes up and down repeatedly. This can be explained by the antiparallel diode in the board Q1 switch. At first, when the capacitor is charging with VSM<9,3 V, the current passes by the antiparallel diode.

When it achieves 9,3 V, the voltage in the capacitor Chigh becomes higher than the voltage on the antiparallel diode anode because the low-side filter capacitor does not charge anymore in the same rate as Chigh. Then, the diode starts to open and Chigh discharges rapidly until it achieves a voltage that allows the antiparallel diode to conduct again.

This creates the up and down cycle that we see: